Fast Super-Resolution Imaging by Single-molecule Localization Microscopy

Dr. Mingshu Zhang

Prof. Pingyong Xu

Key Laboratory of RNA Biology, Institute of Biophysics, Chinese Academy of Sciences

Backgroud

Single-molecule localization microscopy (SMLM), such as PALM, achieves the highest spatial resolution among all the SR imaging methods. However, reconstruction need a large number of raw images, which leads to low temporal resolution. Limited photocontrollable fluorescent protein probes (PCFPs) also restrict the widespread application of SMLM. Prof. Pingyong Xu’s lab are engaged in the development of new super-resolution microscopy technology. Previously, his group developed a new super-resolution method named single-molecule-guided Bayesian localization microscopy (SIMBA). It uses only 100 raw frames to calculate an super-resolution image with a 50 nm spatial resolution, which efficiently increases the temporal resolution of SMLM.

“[We] benefited from the high speed and sensitivity of Prime 95B, and could further increase the temporal resolution of SIMBA with shorter exposure time.

Challenge

To achieve higher temporal resolution of SIMBA, prof. Xu intend to develop imaging techniques that require less raw frames for reconstruction. Another choice is to use camera with higher frame rate and allows shorter exposure time. However, the slow readout speed of the EMCCD camera they used and the suboptimal properties of existing PCFPs hindered them from further increasing the temporal resolution of SIMBA.

Solution

To cover the shortage of EMCCD, they decided to use Teledyne Photometrics sCMOS camera Prime 95B. Combined Prime 95B with pcStar, a new PCFP also developed by Xu lab, they created a new super-resolution method Quick-SIMBA. Quick-SIMBA only needs 50 raw frames (2-5ms exposure) to reconstruct a final image. It could achieve the highest temporal resolution (0.1−0.25 s) with large FOV (76 × 9.4 μm2−76 × 31.4 μm2) among all the SMLM methods.

With Quick-SIMBA, the dynamic movements of the endoplasmic reticulum dense tubular matrix were resolved, which are difficult to recognize by diffraction-limited microscopy. The combination of pcStar and Quick-SIMBA holds great potential for super-resolution imaging of short-term events, such as transcriptional bursts and cell mitosis, or for tracking short-lived proteins in living cells.

Image

Figure1. Quick-SIMBA imaging of the ER structures in live U2OS cells and COS-7 cells. (a) Diffraction-limited image represented by the sum of the first 50 raw frames. (b) Reconstructed images using the Quick-SIMBA algorithm.

Reference

Fast Super-Resolution Imaging Technique and Immediate Early Nanostructure Capturing by a Photoconvertible Fluorescent Protein. M Zhang et.al. Nano Letters 2020 20 (4), 2197-2208